防雨罩使用说明书

Waterproof Cover Manual

产品参数：

■电压范围：220V

■内部工作温度：0°- 36°

■工作环境温度(℃): -20°-  40°

■高: 720mm

■直径: 610mm

■净重: 19KG

■IP指数: IP54

■灯具控制: DMX512

■安装方式:可以平放、侧挂，吊挂进出风弯管可以旋转至任意角度

■功能: 防雨,户外使用

■材质: 透明罩是美国进口的工程塑料制造而成，具有高透光性、抗紫外线、

不发黄、不易破裂等特性。

■信号及电源连接：配有两条2*0.3m²防水信号线和两条3*2.5m²防水电源线，

最大功率2500W.

■使用范围: 200W,230W光束灯或者其他类似体积的灯具。

■保质期：1年

■ voltage range:220V

■Inside Working Temperature (℃): 0°- 36°

■Environment Working Temperature: -20°- 40°

■Height: 720mm

■Diameter:610mm

■N/W: 19KG

■Protection degree: IP54

■Control: DMX512

■Installation: Can be flat, side hanging, hanging, in and out of the wind pipe can

rotate to any angle.

■Function: Raining proof, for outdoor use

■Material: Transparent cover is made by the US imported engineering

plastics with high light penetration, anti-ultraviolet radiation, not

yellow, not easily broken.

■Signal And Power Connections: With two 2*0.3m²waterproof dmx cable and

two 3*2.5m² waterproof power line.,maximum power2500W.

■Suitable: 200W,230W beam lights and similar sized lights.

■Warranty: 1 year

安装方案：

Installation:

- 底座板Base

- 2.将配套胶圈与底座板连接 Fix the matched ring on the base.

- 将连接板两边的固定螺丝取下  Take off the fixed screws on both sides of the connecting board.

- 取出底座的连接板与灯具连接，根据灯具的大小位置调节，快速锁拧紧。

- Connect the board and light, and adjust it according to the light, then screw them tightly.

- 将灯具放置底座中，用原取下的螺丝固定连接

- Put the connecting light and board on the base, and screaw-in the screaws have been taken off.

- 将透明罩与底座连接，用底座蝴蝶扣与透明罩固定

- Put the cover on the base and fix it with the metal button on the base.

- 将配套通风弯口与底座连接Connect the tube on the base.

- 两边安全绳的连接Fasten the metal button and the tube.

- 安装完毕

- Installation OK

- 注意事项Remark：

- 1.透明罩上不能有任何遮挡物

- There should be nothing cover the waterproof cover when in use.

- 严禁带电拆装。

- Do not disassemble the cover with electricity.

- 3.严禁遮挡进排风口，保留空间。

- Do not cover the exhaust outlet and keep some space.